Software Process Used in SwimSafe AI

The development of the SwimSafe AI system involved the integration of multiple software tools, frameworks, and platforms to deliver a real-time drowning detection and alert mechanism. This section outlines the software development workflow and how each tool contributed to the system.

1. Dataset Preparation and AI Model Training (Roboflow + YOLOv8)

- The drowning detection AI model was developed using Roboflow and YOLOv8 (You Only Look Once version 8).

- Using Roboflow, a custom dataset of pool scenarios was uploaded and annotated with labels such as "drowning", "swimming", and "idle".

- Data augmentation techniques were applied to improve model performance under various conditions.

- The dataset was exported in YOLO format and used to train a YOLOv8 model in Python.

- The trained model was then deployed on Roboflow’s hosted inference API, making it accessible for real-time detection via HTTP requests.

2. Flask Web Application (Real-Time Detection and Web Interface)

The core software system was developed using the Python Flask framework, which handles:

a. Video Stream Handling

- A live video feed is captured using a virtual webcam from OBS Studio and processed by the Flask server.

b. AI Inference

- Frames from the video stream are sent to the hosted YOLOv8 model via Roboflow API.

- The returned predictions are parsed to detect "drowning" labels with high confidence scores.

c. Event Triggering

- When a potential drowning incident is detected, the Flask app sends an HTTP request to the ESP32 microcontroller over Wi-Fi to trigger the alarm system (buzzer and LED).

d. Detection Logging and Storage

- Detection events (timestamp, label, confidence score, image snapshot, etc.) are stored in a MySQL database.

- This enables structured and persistent storage of incident history for monitoring and analysis.

e. Web Interface

- The Flask app includes a front-end interface (HTML/CSS with Jinja2 templates) allowing users to:

  - View a live video feed with bounding boxes showing detected individuals.

  - Access a History Page showing past detection logs stored in the MySQL database.

3. MySQL Database – Detection History Storage

- A MySQL database was integrated with the Flask application to manage historical records of drowning detections.

- Each record includes:

  - ID (Primary Key)

  - Timestamp of the event

  - Detected label (e.g., "drowning")

  - Image path or snapshot of the detection

- This database allows for reliable storage, easy querying, and scalability in real-world deployment.

4. Visual Studio Code (VSCode) – Development Environment

- VSCode was the primary development tool for writing Python (Flask app), HTML templates, CSS, and JavaScript for the user interface.

- VSCode was also used for managing virtual environments, handling version control with Git, and debugging code.

5. Arduino IDE – ESP32 Microcontroller Programming

- The Arduino IDE was used to write and upload code to the ESP32 microcontroller.

- The ESP32 connects to Wi-Fi and listens for alerts from the Flask server.

- Upon receiving a signal, it triggers:

  - A passive buzzer to emit an audible alarm.

  - An LED to flash as a visual indicator of emergency.

6. OBS Studio – Virtual Camera for Testing

- OBS (Open Broadcaster Software) was used during development to simulate a real-time camera feed using pre-recorded videos or screen capture.

- This allowed testing of detection logic and system response without needing a physical pool or live drowning scenarios.

Software Workflow Summary:

- OBS Studio streams video to Flask via a virtual camera.

- Flask captures frames and sends them to Roboflow YOLOv8 API for analysis.

- If a drowning condition is detected:

  - Flask logs the event in the MySQL database.

  - Sends an HTTP request to the ESP32 microcontroller.

- ESP32 triggers buzzer and LED for immediate alert.

- A web interface allows users to:

  - Monitor live detections.

  - Review incident history with detailed logs and images.